Measuring volumetric urban design: Compression, multi-level connectedness, and functional mix in transit-oriented developments in Hong Kong

Abstract: High-density cities like Hong Kong are shaped by volumetric urban design, featuring three-dimensional pedestrian networks and integrated metro stations that follow transit-oriented development (TOD) principles—features that conventional measures cannot adequately capture. This study addresses this gap by developing metrics for three core elements of volumetric urban design: compression, multi-level connectedness, and functional mix. Leveraging open-access, high-granularity data on pedestrian networks, buildings, and points of interest, we analyse two major metro station areas in Hong Kong. Our results reveal that 57% of buildings are genuinely mixed-use, with clear vertical zoning—commercial and visitor uses at the podium and residential above. These areas exhibit pronounced spatial compression and high multi-level connectivity, characteristics that traditional two-dimensional metrics systematically overlook or misclassify. Specifically, conventional methods fail to capture up to 91% of spatial compression, 52% of pedestrian connectedness, and misclassify all buildings as mono-functional. By capturing multi-level connectivity, intensified space use, and vertical functional organisation—hallmarks of volumetric urbanism—our approach provides a transferable framework for measuring and understanding complex urban environments. This research offers significant implications for quantitative analysis, urban design, and planning policy, equipping researchers and practitioners with essential tools to quantify and understand high-density, transit-oriented cities.

Keywords: Volumetric city, urban design, transit-oriented development, pedestrian networks, High-density cities

# Introduction

Cities worldwide face the persistent challenge of balancing rapid population growth with limited available land, a dilemma that is especially acute in Asia (Al-Kodmany, 2018; Wang, 2020). In response, urban development has increasingly focused on creating high-density, highly connected, and multi-functional environments to maximise land use efficiency (Harris, 2015).  Recently, the idea of the volumetric city has emerged, capturing the complexity and multi-layered nature of built environments in major Asian cities like Tokyo and Hong Kong (Bruyns et al., 2021; Shelton et al., 2013). The volumetric urban design extends beyond simple verticality, involving the compression and integration of public spaces and buildings to accommodate diverse urban functions within interior and multi-level spaces.

Volumetric urban design builds on earlier urban theories such as new urbanism, compact city, and vertical urbanism (Lin & Gámez, 2018), all of which advocate for dense, mixed-use, and walkable environments in response to car-oriented development and urban sprawl. These ideas have shaped contemporary planning movements, including the 15-minute city (Rao et al., 2024). Specifically, vertical urbanism extends these principles to high-rise, space-efficient environments, evolving from the modernist focus on skyscrapers to more integrated and socially engaging three-dimensional systems that connect infrastructure, buildings, and public spaces across vertical and horizontal layers (Harris, 2015). While verticality has characterised large-scale developments in global cities such as New York, Dubai, and Chicago, recent research highlights the volumetric city as a distinct model, particularly evident in Asian contexts (McNeill, 2020). In Hong Kong, a volumetric development is characterised by highly compressed, multi-layered, and internalised environments that integrate buildings, pedestrian networks, privately owned public spaces, and transit-oriented development (TOD) principles (Shelton et al., 2013). Volumetric design is thus best understood through key elements such as density, compactness, diversity, and connectedness (Lehmann, 2016), which capture both the physical compression of space and the integration of diverse functions across multiple levels (Bruyns et al., 2021). Despite increasing attention, most current studies rely on narrative and qualitative approaches to understand these complex, multi-layered urban forms (Lehmann, 2016).

Using Hong Kong as the case city, this paper aims to measure the volumetric urban design, with a focus on compression, multi-level connectedness, and functional mix. The first defining feature of volumetric design is the integration of three-dimensional pedestrian networks that connect multiple layers both vertically and horizontally (Sun et al., 2021). Public sectors and private developers provide extensive elevated pedestrian systems with public access, increasing street connectivity, reducing detours, and facilitating smooth pedestrian movement among buildings (Tan & QL Xue, 2014). This principle also underpins the urban design approach that creates highly connected, multi-layered, and internalised environments through the seamless integration of buildings, open spaces, and streets (Villani et al., 2022). Despite growing recognition of these multi-layered characteristics in high-density cities (Harris, 2015), most studies still rely on a figure-ground approach, focusing on mono-functional land plots, street centroid lines, or building footprints. Recent work has introduced new methods to capture the vertical complexity, such as function composition using cadastral data (Dovey & Pafka, 2017), multi-scale density (Pafka, 2023), and compression via building volumes (Wang et al., 2022). However, these metrics are typically limited to small areas and lack broader applicability for cross-context comparisons. There remains a lack of a generalisable framework for understanding the principles and measuring the volumetric urban design.

The second defining feature of volumetric urban design in Hong Kong is the integration of the Mass Transit Railway (MTR) system, which follows transit-oriented development (TOD) principles. Hong Kong’s reliance on the metro system is evident: nearly half the population lives within 500 metres of a station, and 70% of riders walk to and from the metro, contributing to an average daily ridership of 4.69 million (Sun et al., 2021). This TOD approach promotes high density, mixed land use, and enhanced connectivity within station catchment areas. In addition, seamless integration of pedestrian networks with metro stations connects multi-level urban interfaces, supporting both mobility and urban cohesion (Villani et al., 2022). The expansion of underground and elevated spaces enables the vertical stacking of urban functions, creating environments that extend above and below ground (Zacharias & He, 2018; Zhang & Chiaradia, 2022).  Metro stations are directly linked to commercial complexes and residential towers through a comprehensive three-dimensional pedestrian network, including footbridges, tunnels, elevators, and escalators. Elevated and underground walkways connect podiums, shopping malls, and towers, generating large, multi-layered volumes and enhancing walkability. Many internal spaces, privately owned yet publicly accessible, are integrated with metro stations, providing climate-controlled, open spaces that serve both commuters and the public (Shelton et al., 2013). The ongoing expansion of the metro and its integration with property developments has fostered creative, pedestrian-oriented urban design, making three-dimensional connectivity a hallmark of the city’s design fabric. However, the complex interactions between multi-level connections, spatial organisation, and TOD in the volumetric design warrant further investigation.

This paper measures three core attributes of volumetric urban design in Hong Kong: compression, connectedness, and functional mix. It addresses a critical gap in measuring multi-layered developments by proposing an operationalised and replicable method, using open-access, fine-grained data. We applied and validated these new measures in areas around metro stations, which are featured as typical volumetric environments and TOD contexts, to confirm their effectiveness. Furthermore, we examined how multi-level pedestrian networks connect metro stations and surrounding buildings by expanding connectivity, compressing space, and redistributing functions vertically and horizontally. This research advances the spatial understanding of complex, multi-layered urban forms and provides novel tools for researchers and practitioners to analyse volumetric urban design in high-density cities.

# Measuring volumetric urban design

As this study is primarily concerned with the development and application of new measures, its focus is methodological. While methodological discussions often raise questions of ontology and epistemology, this paper does not engage with those debates. Instead, we adopt a pragmatic approach, concentrating on the operationalisation of concepts for urban science and practical urban analysis. Although we acknowledge the value of complexity and relational theories (e.g., assemblage thinking in walkability) (Dovey & Pafka, 2020), our aim is to provide useful, concrete tools with measurable outcomes. This necessarily involves some reductionism, as we strive to identify clear patterns that can be quantified and applied in real-world urban contexts.

Our literature review focuses on quantifying origins, concepts, and evolution of three key elements of volumetric urban design: compression, connectedness, and functional mix. These measures build upon the well-established concepts of density, connectivity, and mix (Table S1), which serve as foundational elements across various urban theories, particularly those related to urban design and urban form.

### 2.1 Compression

Compression reflects the spatial configuration and geometric morphology of buildings (Bruyns et al., 2021). It builds on the concept of density, a widely recognised term in urban planning and design (Marshall et al., 2019). Density broadly encompasses both social and spatial elements, with common indicators such as population density, building coverage ratio, and floor area ratio measured within specific units (e.g., census tracts or neighbourhoods). However, the validity of traditional two-dimensional density measures is often questioned in volumetric settings, since changing the unit size around metro stations can lead to significant variations and measurement biases.

In volumetric settings, compression refers to the intensity of functions packed within building spaces. Buildings that accommodate more functions in closer proximity are likely to generate greater opportunities for interaction, assuming other factors remain constant. Thus, building floor size or volume (Wang et al., 2022), as well as spatial arrangement and proximity to surrounding buildings, are essential considerations. Bruyns et al. (2021) argued that building-level units are more appropriate for measuring density in highly compressed environments, using straight-line or walking distances. They introduced the physics concept of volume to incorporate three-dimensional information, moving beyond traditional two-dimensional building footprints (Shelton et al., 2013). Gravity-based network analysis further enhances the measurement of compression by accounting for spatial interaction opportunities between buildings and the impedance along streets (Bruyns et al., 2021).

However, simple measures of building volumes fail to capture the functional configuration and attractiveness of spaces, and do not adequately reflect use intensity or interaction opportunities. For example, residential and commercial buildings with similar volumes would have vastly different functional configurations, but can not be captured by volume alone. Therefore, research must integrate three-dimensional information and function-related attributes, such as destinations and attractions, into compression measures to better capture the complexity of volumetric urban design.

### 2.2 Multi-level connectedness

Wider pedestrian pathways, well-designed interfaces, and effective separation from motorised traffic are essential for enhancing the network’s capacity to support pedestrian movement and circulation. In high-density cities, streets are often compressed due to limited land and the drive to maximise building floor areas, making sidewalk width a critical factor in determining the quality of connectedness. Narrow sidewalks, especially when combined with high pedestrian volumes, can restrict walking speed and diminish the overall pedestrian experience (Zacharias, 2021).

With the metro system as the central fabric, connectedness links public spaces and destinations across multiple layers, as well as integrating interior and exterior spaces that often extend from metro stations. Connectedness, therefore, extends beyond basic connectivity by incorporating horizontal intensity dimensions, while also addressing vertical connections. Vertical connections, both above and below ground, are particularly important, as they enable seamless transitions in dense urban environments. Vertical pedestrian facilities, such as staircases, escalators, elevators, footbridges, and underpasses, play a crucial role in facilitating movement within these complex settings (Bruyns et al., 2021; Li et al., 2023). Unlike traditional connectivity measures based on road centrelines, an approach centred on pedestrian-accessible spaces highlights the importance of three-dimensional pedestrian networks for calculating and improving connectedness in high-density contexts (Zhao et al., 2021).

### 2.3. Functional mix

The functional mix represents the diversity of horizontal and vertical land uses, illustrating how different categories are integrated within blocks and neighbourhoods (Bruyns et al., 2021). Measures of functional mix originated from land use diversity studies in Western cities, which are characterised by low-rise, mono-functional buildings, and typically use indicators such as land use entropy, dissimilarity, and mixed-use indexes (Wandl & Hausleitner, 2021). In contrast, Asian cities, constrained by land shortages, have a long-standing tradition of integrating multiple uses within single buildings. A single building might combine residential and commercial uses on different floors, with office spaces sandwiched between grocery stores and apartments (Lau et al., 2005). TOD principles further encourage this, with metro underground spaces often serving as podiums that host shops, leisure spaces, restaurants, and several high-rise residential buildings above. Such vertical layering challenges traditional methods that rely on ground-floor or parcel-level classifications, which cannot capture vertical variations in urban functions. Additionally, conventional measures of land use mix face overlapping sub-categories of functions (Dovey & Pafka, 2017), complicating comparison across multiple datasets.

Ternary graphs, which visualise the composition of three functional categories (e.g., live, work, visit), offer a promising way to capture the complexity and synergy of urban functional configurations, especially given classification challenges (Dovey & Pafka, 2017). Using a triangle highlights the interconnections between functions rather than just their presence. Mapping these relationships at multiple scales can reveal streets or neighbourhoods as unique blends of mixes, facilitating the interpretation and diagnosis of functional diversity. While fine-scale datasets like cadastral data remain limited in scope, the widespread availability of point of interest (POI) data in high-density Asian cities acts as an alternative and enables detailed, large-scale analysis of functional mix, including spatial and floor-level information (Higgins et al., 2021). POI data has proven effective in measuring both horizontal and vertical land use mix (Zhang et al., 2025), and can illustrate parcel-level diversity beyond traditional land use categories (Wandl & Hausleitner, 2021).

Based on the literature review, we proposed an operationalised and replicable framework for measuring the volumetric city to assess volumetric design—encompassing compression, connectedness, and functional mix—using open-access data and generalizable approaches (Fig. 1).

[Insert Fig. 1 here]

# Method

## Hong Kong and the study sites

Hong Kong is a high-density city, with built-up areas exceeding 20,000 people per square kilometre (Census and Statistics Department of Hong Kong, 2024, 2025). Renowned for its TOD practice, the city has over 100 metro stations, with most urban development situated nearby. To develop and test our measures, we selected Central and Tsim Sha Tsui (TST) as study sites (Fig. S1). We aimed to develop operationalizable and generalizable measures using fine-grained, open-access data to capture volumetric urban design, rather than applying these measures broadly for comparison across multiple areas. Central and TST were chosen as typical cases for their varied historical development, diverse building forms (e.g., from colonial-era structures to modern high-rises on reclaimed land), representative volumetric design (e.g., extension of networks in three dimensions), and the implementation of TOD concepts.

Central, located on the northern shore of Hong Kong Island, is the CBD and home to the headquarters of multinational companies. It features the city’s most extensive three-dimensional pedestrian network, including 8 km of elevated walkways that link Central and Sheung Wan. Served by three metro stations—Central, Hong Kong, and Sheung Wan—with 26 exits and extensive underground networks integrated with property development. The area around Hong Kong Station, developed in the 1990s, was renowned for its property development, incorporating TOD concepts. Central exemplifies high-density, volumetric urban design characterised by dense, multi-functional developments and multi-layered pedestrian connections.

Tsim Sha Tsui, located at the tip of the Kowloon Peninsula, is a major tourist and commercial hub known for its shops, hotels, and restaurants. The area has two metro stations (TST and TST East) with a total of 26 exits, allowing seamless transfers between the stations and surrounding destinations. Extensive underground pedestrian networks, developed in tandem with commercial growth and expanded by private developers, facilitate pedestrian movement below ground (Zacharias & He, 2018). Recent projects near TST East station, such as Victoria Dockside, follow TOD principles and exemplify multi-functional, high-density urban design.

## Dataset and processing

The workflow of this study is shown in Fig. 2. We collected POI data from Gaode Map, which offers more comprehensive coverage of geographical entities in Hong Kong than official or other third-party sources. Data accuracy was verified by randomly sampling POIs and cross-checking with field observations and alternative datasets (e.g., Google and Foursquare). The Gaode dataset includes 23 major classifications, with each POI containing longitude, latitude, and exact floor information. POI categories located outside buildings (e.g., parking, transportation) were excluded, resulting in 10 retained categories for analysis (e.g., catering, hotels, tourist attractions). To address positional inaccuracies, we manually corrected deviated POIs to ensure alignment with building footprints.

[Insert Fig. 2 here]

We used two primary data sources to map multi-level pedestrian networks. First, we drew on a comprehensive geodatabase of Hong Kong’s three-dimensional pedestrian networks (Sun et al., 2021), which classifies 23 types of pedestrian paths into three height levels: ground-level sidewalks (Level 0), underground paths (Levels L3–L1, e.g., metro concourses), and elevated walkways (Levels 1–2, e.g., footbridges, interior public pathways). This dataset notably includes interior pedestrian networks with public access, such as privately owned public spaces within podiums and atria. Second, the Hong Kong Lands Department’s iB1000 topographic dataset provided detailed street information, including boundaries between buildings, motorised lanes, and outdoor sidewalks (e.g., stairs, alleyways, footbridges), as well as building footprints and heights. Building complexes with low-rise podiums and high-rise towers are common in Hong Kong, and the iB1000 dataset captures these forms in detail. We estimated the number of building floors by dividing building height by an average floor height of 3.4 meters (Li & Tsang, 2008).

Data processing was involved in several steps. First, sidewalk boundaries were identified, and sidewalk widths were manually extracted in GIS based on street boundaries and building footprints. Since widths vary along each segment, we used the narrowest point to represent the minimum pedestrian movement capacity for each segment. For pedestrian network types not captured in the datasets, such as indoor, underground segments, and crossings, we referenced Hong Kong’s design criteria for width assessment (Table S2). Second, floor information was extracted from POI address text using Python, allowing each POI to be assigned to a specific floor. Descriptive statistics are provided in Table S3.

## Measures for volumetric urban design

### 3.3.1 Compression

Compression was defined as the aggregated interaction opportunities between a building and its nearby buildings, mediated by multi-level pedestrian networks and accounting for distance decay. We applied a gravity-based model, commonly used to measure spatial interactions (Thill et al., 2011). In our model, the number of POIs is weighted to reflect the interaction opportunity and attraction between buildings. The compression level for each building was calculated as the number of POIs divided by building volume, reflecting the assumption that a higher density of POIs within fewer floors indicates greater functional intensity and economic activity. Spatial impedance was measured along the three-dimensional pedestrian networks, with a negative exponential specification to calibrate walking trips (Kwan, 1998). The compression level of a location i is Comi:

Comi=jPj* RdijFi    (1)

Rdij=e-tij2β    (2)

Where i is the building, and j is the nearby building of i, Pi is a measure of the number of POIs within the building i, Fi is a measure of the floor number of the building j. Rdij is the impedance along the pedestrian network, tij measures the spatial impedance between i and j, and e is a parameter of spatial friction. The interaction level between buildings exhibits distance decay effects, and we followed Handy and Niemeier (1997) with β equal to 0.1813 to calibrate walking trips. The analysis of compression used the accessibility toolbox developed by Higgins (2019) in ArcGIS Pro. We followed the nearest distance assumption in the compression modelling for methodology consistency. It should be noted that several buildings have multiple entries, and the nearest distance between street networks and buildings was not necessarily aligned with actual pathways for users.

### 3.3.2 Connectedness

We defined connectedness as the indicator measuring the public interface that can be reached within walking distance from each building. The measure is a proxy for the walkable and permeable interface that pedestrians encounter. The calculation refers to the aggregation of walkable space of different street segments in catchment areas; a higher score indicates that a larger street space accommodates pedestrians. The walkable space of each segment is the length of each street segment multiplied by its width. We calculate the catchment area of the location i based on the three-dimensional pedestrian networks. The formula is as follows:

Coni=i=0jWj*LjSi    (3)

Where Coni is the captured connectedness level near the building i, and Wj is the narrowest width of the street segment j within the catchment area, and Lj is the length of the street segment j. Wj*Lj is the walkable space in different layers. In addition, Si is the catchment area expanded from building i along street segments j.

Edge effect in this study could arise from two sources. First, natural barriers such as the sea, protected areas, and country parks restrict urban expansion, resulting in a lack of built environment information for areas extending beyond these boundaries (e.g., waterfront buildings facing the sea). Second, edge effects occur where the study area borders developed regions not included in the dataset, potentially resulting in incomplete information near the boundaries. To address this, we excluded catchment areas that overlapped with unbuilt areas (e.g., sea) and built areas outside the study boundary. Catchment areas for connectedness and compression were defined as 400 m network-based distances from each building centroid, reflecting the typical walking distance preferred by Hong Kong residents (Audi et al., 2010).

### 3.3.3 Functional mix

We assessed the functional mix of each building across three categories—live, visit, and work—using a ternary graph (Dovey & Pafka, 2017), which graphically represents the composition of three variables in a two-dimensional triangle. Each point’s position on the plot corresponds to its trilinear (barycentric) coordinates, enabling intuitive visualisation of functional composition. Functional mix was interpreted using the parallel line method, where lines parallel to each triangle edge indicate constant values for the corresponding function at each vertex. Each component is 100% at its vertex and decreases linearly to 0% at the opposite side. Based on these coordinates, we classified buildings into 100 functional mix levels. Buildings located on the triangle’s edges were considered mono-functional (pure live, work, or visit). In this study, “live” refers to residential estates and hotels; “work” encompasses business, office, government, institutional, educational, and financial uses; and “visit” includes catering, daily services, retail, healthcare, and sports and leisure facilities.

We developed two building-level functional mix indicators: overall and vertical functional mix. The overall functional mix captures all POIs within a building, while the vertical functional mix focuses on POIs located above the ground floor. This distinction is important, as upper floors in Hong Kong’s city centre are frequently used for non-residential functions and are connected by decks or bridges (Shelton et al., 2013). For the vertical mix, housing-floor POIs were identified using POI data, supplemented with online sources and site surveys. Using the ternary plot, we visualised the functional mix levels for each building, categorising both overall and vertical functions as live, work, or visit. A building was classified as “mixed” if it contained POIs from more than one category; otherwise, it was shown in black. Notably, these indicators reflect the functional diversity within buildings, rather than the intensity of use.

## Evaluating the volumetric design measures

### 3.4.1 Comparing with two-dimensional measures

We need to know how proposed measures outperform two-dimensional indicators and work as benchmarks in representing volumetric urban design. The intensity of compression, connectedness, and functional mix depend on capturing three-dimensional information, such as multi-level networks and floor-based information (e.g., networks, building floors, and floor-level functions). We examine the measures with and without three-dimensional information (e.g., ground-floor networks, building footprint, and mono-functional building) on the study sites. Examination of the change in the measures can verify the bias of two-dimensional measures.

### 3.4.2 What-if scenario analysis

We conducted a what-if scenario analysis for a site in the Central area, connected by elevated walkways, to examine the impacts on volumetric design outcomes. Specifically, we evaluated changes in connectedness if the elevated pedestrian network were removed. We also assessed how elevated walkways influence compression by supporting additional interactions above ground. For the functional mix, we analysed whether elevated walkways create an additional “ground floor” for pedestrians, thereby altering the vertical functional arrangement of buildings.

Noted functional mix operates at multiple scales. The elevated walkways would provide sheltered, separated pedestrian routes, connecting individual buildings to larger complexes. As a result, a building’s functional mix may reflect the functions of adjacent connected structures. To investigate this, we examined how elevated walkways redefine the multi-scale functional mix by linking neighbouring buildings. The analysis involved two steps: first, we grouped directly connected buildings (e.g., atrium, podium, and tower) as single complexes. Second, we calculated each building’s function mix by incorporating a weighted average of the functions of nearby buildings connected via walkways within a 50-meter radius. For example, if a building was linked to two others, its functional mix was determined by the average of all three buildings’ functions.

Additionally, t-tests were used to compare the proportion of visit-related functions on floors connected by walkways (below the third floor) with those on higher floors within the same buildings. Paired t-tests assessed how multi-level pedestrian networks influenced vertical functional variation by comparing functional components across different floors.

# Results

## Volumetric design measures

### 4.1.1 Compression

Figures 3 (a) and (b) illustrate the compression levels in the Central and TST areas. In Central, building compression ranged from 0 to 100.25. High compression was clustered in the western coastal zone; the mid-levels (shown in the southern part of the figure) exhibited lower compression, likely due to the predominant residential use. The eastern side also showed low compression, possibly due to the presence of large, single-owner buildings, such as public institutions and government buildings. Areas of high compression roughly corresponded with metro stations, reflecting the long-standing clustering of high-intensity commercial buildings and office towers in Hong Kong’s CBD.

Compression levels in TST ranged from 0 to 151.77. Higher compression was observed in the northern parts, while the southern and coastal areas showed lower levels. This pattern is likely due to lower building density and fewer connections in coastal areas, reducing interaction opportunities between buildings. Notably, highly compressed zones in TST did not correspond to metro station catchment areas, suggesting that there is no clear alignment between TOD and volumetric intensity in terms of compression within TST.

[Insert Fig. 3 here]

### 4.1.2 Connectedness

Figures 3(c) and 3(d) present connectedness levels for Central and TST. In Central, connectedness ranged from 0.01 to 0.09, indicating that 1% to 9% of the spaces around buildings were walkable relative to the total area. The highest connectedness was found in the central core, decreasing outward, likely due to the extensive multi-level pedestrian networks provided by MTR and private developers. Highly connected areas were concentrated around Central Station (located in the east), but not at Sheung Wan Station (located in the west).

TST exhibited higher and more consistent connectedness, ranging from 0.03 to 0.10. High connectedness was clustered in the north, likely due to small block sizes and a dense network of lanes, enabling diverse pedestrian routes. Outer areas, especially along the waterfront promenade, showed lower connectedness, reflecting limited pedestrian access at the edge. Additionally, these highly connected zones did not overlap with the locations of metro stations.

### 4.1.3 Functional mix

Table 1 summarises the building-level overall and vertical functional mix results. In Central, 2,108 out of 2,566 buildings contained POIs, indicating assigned functions. Of these, 1,402 buildings (54.64%) exhibited an overall functional mix, defined as having at least two functional components of POIs. In TST, 663 of 771 buildings had specified functions, with 511 (66.28%) showing a functional mix. For the vertical functional mix, 1,088 buildings in Central and 663 in TST had POIs located above the ground floor. Among them, 567 buildings in Central (22.10%) and 326 in TST (42.28%) exhibited a vertical functional mix. Overall, across both study areas, 57.33% of buildings demonstrated an overall functional mix, while 26.76% exhibited a vertical functional mix.

[Insert Table. 1 here]

Figure 4 visualises and presents the distribution of building-level overall and vertical functional mix. In Central, the predominant mix was a combination of work and visit functions within the same building, especially in coastal areas, reflecting the concentration of commercial and administrative entities around the harbour. Mixing of residential functions with work or visit uses was less common, with residential buildings primarily located in mid-level areas (in the south). The coastal zone showed a strong work-visit mix, while a visit-live mix was more typical in the mid-level areas. Buildings in the northeast were mostly mono-functional, often due to single ownership by companies or institutions. The overall functional mix level did not correspond with the metro station catchment area. Vertically, mixed-use buildings in Central were mainly concentrated along the coast, while buildings in the mid-levels were predominantly residential in their vertical arrangements.

Both overall and vertical functional mixes were more prevalent in TST. Many buildings, integrating high proportions of work and visit functions, exhibit a high degree of functional mix. For vertical functional mix, visit-related uses are also common on upper floors, together with residential functions such as hotels and hostels. Thus, the results quantitatively verify the typical podium-tower form characteristic of Hong Kong in TST, with commercial uses at the base and residential uses above. These results visualise the prevalence of multi-functional buildings and their distribution. However, the work component was relatively low, likely because retail and commercial activities are primarily clustered to serve the local industry rather than to accommodate office or business uses.

[Insert Fig. 4 here]

## 4.2 Evaluation of the new measures

### 4.2.1 Comparison with two-dimensional measures

Figures S2(a) and S3(a) compare and illustrate the compression level based on two-dimensional data. Although the absolute value of compression changed, the relative ranking of compression levels remains stable. It highlights the effectiveness of well-connected ground-level pedestrian networks in mitigating the loss caused by elevated or underground connections, particularly in terms of compression. Figures S2(b) and S3(b) illustrate the estimated bias of the connectedness level using two-dimensional information, which only reflects the ground-level networks. We used the same intervals in the visualisation to ensure comparability with the results presented in Section 4.1. The impacted areas cluster around the two metro station areas in Central, with up to 51.51% of the connectedness level unaccounted for (Figure S2(c)). In contrast, the impact on TST was more evenly distributed. Furthermore, the results highlight that the connectedness variation was relatively limited for ground-level networks. Furthermore, Figures S2(d) and S3(d) illustrate the building function based on the land-use map. Table S4 presents the building function patterns in the Central area, illustrating the dominant live and visit land uses. However, land use data neglects the extensive presence of mixed-use buildings and overestimates the live function. These highlight that the neglect of nuanced building configurations has led to estimation bias and oversimplified spatial layouts.

### 4.2.2 Impacts of pedestrian networks on volumetric design

The “what-if” scenario results provide insights into how multi-level pedestrian networks affect compression, connectedness, and functional mix. We focused on areas in Central connected by elevated walkways, which form one of the largest grade-separated pedestrian systems globally, linking office towers, commercial complexes, metro stations, and privately-owned public spaces.

Figure 5(a) shows that elevated walkways significantly enhance compression levels for buildings and their interactions above ground. The spatial impact varied widely, with walkway contributions to building compression ranging from 0% to 91.67%. This demonstrates that elevated walkways function as an additional “ground” level, supporting the vertical stacking and concentration of building activities. Figure 5(b) illustrates changes in connectedness if elevated walkways were absent. The removal of these networks would reduce connectedness by 5.90% to 35.29% for affected buildings, highlighting the critical role of elevated pedestrian infrastructure.

Figure 5(c) details floor-level functions in walkway-connected buildings. Visit-oriented functions dominate podiums and atria, while upper floors are primarily for work. Paired t-tests (Table 2) confirm that floors directly linked to elevated walkways have a significantly higher proportion of visit functions. Thus, elevated pedestrian networks create an effective additional ground floor, supporting shopping and service activities in podium and atrium forms. Figure S4(c) presents the multi-level functional mix for buildings connected by elevated walkways. Assuming organic connections via sheltered walkways, the results show that these networks facilitate a multi-scale functional mix, effectively integrating multiple buildings into unified complexes. In a word, elevated walkways underpin functional variation vertically and horizontally, and underpin compression and functional mix embedded in volumetric design and TOD.

[Insert Table. 2 here]

[Insert Fig. 5 here]

# Discussion and conclusion

This study presents an operationalised and replicable framework for measuring volumetric urban design, focusing on compression, connectedness, and functional mix. By developing and applying these new measures, we reveal the unique spatial logic and functional organisation of volumetric urban environments, highlighting vertical zoning and dense, interconnected spaces that traditional two-dimensional approaches overlook. Our analysis demonstrates that conventional metrics cannot capture the complexity of Hong Kong’s high-density, transit-oriented settings, which are defined by multi-level pedestrian networks and integrated metro stations following TOD principles. These findings underscore the need for new frameworks to guide urban analysis, design, and policy in high-density cities shaped by volumetric development.

Specifically, our findings highlight that multi-level pedestrian networks and metro stations support connectedness, compression, and functional mix in volumetric design. These results are consistent with previous qualitative observations that podium decks serve as new "grounds" for towers, while bridges and ramps connect multiple layers to create multi-level environments around station catchment areas (Shelton et al., 2013; Tan & QL Xue, 2014). Often realised as privately owned public spaces, multi-level walkways not only expand connectivity and enable denser urban forms but also enhance the spatial configuration of shopping and service sectors. Our study further demonstrates that elevated walkways alter the functional mix at multiple scales by connecting diverse building types—such as atria, podiums, and towers—across different levels. Moreover, since the principles underlying these three volumetric elements closely align with TOD strategies in high-density cities, multi-level pedestrian networks around metro stations foster integrated urban fabrics characterised by both high volumetric intensity and TOD features.

Our compression measure defines the opportunity to access destinations from a given building, including those reachable via connected pedestrian networks. Our findings reveal how urban functions cluster and interact in high-density environments. The spatial distribution of compression highlights how building spaces function as nexuses between spatial form and urban activity, and shows how small areas (e.g., 5-minute neighbourhoods) are functionally organised. Our analysis highlights the crucial role of three-dimensional pedestrian networks in amplifying the intensity of these clusters, rendering compression a valuable proxy for estimating potential users, pedestrian flows, and overall urban vibrancy at a fine-grained level in the volumetric city.

Our findings indicate that the combination of permeability and street width determines interface catchments, which in turn influence the proportion of walkable public areas. These aggregated catchments determine the capacity for pedestrian movement, spatial integration and interaction from a street design perspective. In the two case sites, only 1% to 10% space relative to catchment areas was open to flexible and efficient movement for pedestrians, highlighting the spatial constraints in dense environments like Hong Kong. Given these limitations, expanding street links or width at ground level is rarely feasible. Instead, multi-layered pedestrian networks—developed by both public and private sectors—have become essential for increasing route options and overall connectedness. This approach diversifies access and improves station catchment areas, and supports successful TOD implementation. In such cases, differences in connectedness levels between areas primarily reflect the extent of vertical network expansion, highlighting the proactive role of planners, designers, and developers in overcoming land-use constraints by creating privately owned public spaces.

This study introduces fine-grained measures of both overall and vertical functional mix. The results show that functional mix exceeds 50% within individual buildings and is over 20% in vertical above-ground spaces, providing strong quantitative evidence of evolving land use and architectural strategies in Hong Kong’s city centre. This scale of functional mix extends beyond single land plots, spanning from colonial-era buildings to modern complexes (Shelton et al., 2013). It maximises daily service provision within blocks or even single buildings. Our findings confirm the prevalence and visualise the distribution of combined work and visit functions, reflected in the high concentration of services, shops, and restaurants (Zacharias & He, 2018).  The observed vertical functional mix further illustrates how activities and spaces are organised through innovative building typologies (e.g., podiums and atria), as well as the adaptive reuse of older structures in Hong Kong with the advancement of vertical connections (Shelton et al., 2013).

This research has several limitations. First, while the use of POIs enabled fine-scale mapping of building functions and supported large-scale analysis, it did not capture the actual attractiveness or intensity of these destinations. Future studies should consider more detailed datasets that account for variations in size, usage patterns, and amenities. Second, there are limitations in measuring connectedness. Although we included pedestrian network widths, some categories could not be measured due to data constraints, which may affect the accuracy of estimated widths. Additionally, our network-based analysis did not account for travel time friction due to slope in the connectedness calculation. Third, compression, connectedness, and functional mix may vary over time, making them difficult to capture with static datasets. Finally, compression calculations did not factor in actual floor areas or the specific routes from building entrances to POIs, which would provide a more nuanced understanding of spatial interactions.

This study also has notable strengths. It provides robust and generalisable methods for measuring urban design using fine-grained, openly accessible data. This approach challenges conventional notions of functional mix, building density, and connectivity. Volumetric design emerges as a human-centred strategy that facilitates pedestrian movement in high-density environments by offering climate-controlled, vehicle-free, safe, and efficient pathways within city centres. Additionally, it serves as an integrative approach that connects urban fabrics across underground metro systems, ground-level spaces, and above-ground developments, enhancing the vibrancy of volumetric developments.

While this study focused on the Hong Kong case, careful consideration was given to volumetric design practice, with implications for other high-density cities in Asia. As cities become increasingly multi-layered, our replicable methodology, based on open-access data, overcomes the limitations of traditional two-dimensional measures. This approach supports large-scale, comparative analyses of urban form and can be readily applied to other high-density contexts, enabling cross-city comparisons and deeper exploration of the economic, social, and behavioural impacts of volumetric development.

# Reference

Al-Kodmany, K. (2018). The vertical city: a sustainable development model. WIT press.

Bruyns, G. J., Higgins, C. D., & Nel, D. H. (2021). Urban volumetrics: From vertical to volumetric urbanisation and its extensions to empirical morphological analysis. Urban studies, 58(5), 922-940.

Census and Statistics Department of Hong Kong. (2024). Land area and gross floor area of private real estate development projects by status of project. https://www.censtatd.gov.hk/en/web_table.html?id=615-73044

Census and Statistics Department of Hong Kong. (2025). Population Estimates. https://www.censtatd.gov.hk/en/scode150.html

Dovey, K., & Pafka, E. (2017). What is functional mix? An assemblage approach. Planning Theory & Practice, 18(2), 249-267.

Dovey, K., & Pafka, E. (2020). What is walkability? The urban DMA. Urban studies, 57(1), 93-108.

Harris, A. (2015). Vertical urbanisms: Opening up geographies of the three-dimensional city. Progress in Human Geography, 39(5), 601-620.

Higgins, C. D., Nel, D. H., & Bruyns, G. (2021). Measuring mixed-use intensity in volumetric cities: density and interaction. ISUF 2020 Virtual Conference Proceedings,

Kwan, M. P. (1998). Space‐time and integral measures of individual accessibility: a comparative analysis using a point‐based framework. Geographical analysis, 30(3), 191-216.

Lehmann, S. (2016). Sustainable urbanism: towards a framework for quality and optimal density? Future cities and environment, 2, 1-13.

Li, D. H., & Tsang, E. K. (2008). An analysis of daylighting performance for office buildings in Hong Kong. Building and environment, 43(9), 1446-1458.

Li, M., Xue, F., & Yeh, A. G. (2023). Bi-objective analytics of 3D visual-physical nature exposures in high-rise high-density cities for landscape and urban planning. Landscape and urban planning, 233, 104714.

Lin, Z., & Gámez, J. L. (2018). Vertical urbanism: designing compact cities in China. Routledge.

Marshall, S., Gong, Y., & Green, N. (2019). Urban compactness: New geometric interpretations and indicators. The Mathematics of Urban Morphology, 431-456.

McNeill, D. (2020). The volumetric city. Progress in Human Geography, 44(5), 815-831.

Pafka, E. (2023). Modelling, mapping and measuring urban densities. In The Routledge Handbook of Urban Design Research Methods (pp. 460-469). Routledge.

Rao, F., Kong, Y., Ng, K. H., Xie, Q., & Zhu, Y. (2024). Unravelling the Spatial Arrangement of the 15-Minute City: A Comparative Study of Shanghai, Melbourne, and Portland. Planning Theory & Practice, 1-23.

Shelton, B., Karakiewicz, J., & Kvan, T. (2013). The making of Hong Kong: from vertical to volumetric. Routledge.

Sun, G., Webster, C., & Zhang, X. (2021). Connecting the city: a three-dimensional pedestrian network of Hong Kong. Environment and Planning B: Urban Analytics and City Science, 48(1), 60-75.

Tan, Z., & QL Xue, C. (2014). Walking as a planned activity: Elevated pedestrian network and urban design regulation in Hong Kong. Journal of urban Design, 19(5), 722-744.

Thill, J.-C., Dao, T. H. D., & Zhou, Y. (2011). Traveling in the three-dimensional city: applications in route planning, accessibility assessment, location analysis and beyond. Journal of Transport Geography, 19(3), 405-421.

Villani, C., Talamini, G., & Xue, C. Q. (2022). Pedestrian access to transit in evolution: unfolding the spatialization of rapid-transit planning. Journal of Urban Design, 27(6), 669-691.

Wandl, A., & Hausleitner, B. (2021). Investigating functional mix in Europe's dispersed urban areas. Environment and Planning B: Urban Analytics and City Science, 48(9), 2862-2879.

Wang, W. (2020). Everyday practice in the high density, volumetric Hong Kong: Ambiguity, intensity and life between interfaces. Cities, 96, 102462.

Wang, Z., Yuan, H., Wu, M., Deng, X., Tang, Y., Shao, J., & Zeng, Q. (2022). Urban compactness: Applying a three-dimensional model to strategic region of metro station. Transportation Research Part D: Transport and Environment, 113, 103541.

Zacharias, J. (2021). Pedestrian dynamics on narrow pavements in high-density Hong Kong. Journal of Urban Management, 10(4), 409-418.

Zacharias, J., & He, J. (2018). Hong Kong’s urban planning experiment in enhancing pedestrian movement from underground space to the surface. Tunnelling and Underground Space Technology, 82, 1-8.

Zhang, E., Hou, J., & Long, Y. (2025). The form of China’s urban commercial expansion in the digital era. Nature Cities, 1-11.

Zhang, L., & Chiaradia, A. J. (2022). Walking in the cities without ground, how 3d complex network volumetrics improve analysis. Environment and Planning B: Urban Analytics and City Science, 49(7), 1857-1874.

Zhao, J., Sun, G., & Webster, C. (2021). Walkability scoring: Why and how does a three-dimensional pedestrian network matter? Environment and Planning B: Urban Analytics and City Science, 48(8), 2418-2435.